PICC1024.GIF and PICC800.GIF

These are a set of photos (both 1028x768 & 800x600) showing

the end resonator (or extra coil) in a small Tesla Magnifier.

The coil is not really visible in this scan, but it consists of

a six inch diameter sealed PVC coil form wound with about 900

turns of #21 magnet wire. The winding length is about 28 inches

and the coil is topped with a 20 inch toriod discharger. This

coil is standing up on a three foot high insulator, it has no

primary or tank circuit, and it is nearly five feet away from

the nearest coil in the system. The extra coil is base fed by

transmission line. The bright area in the bottom right of the

scan is lit by the spark gap system, in this light the top two

turns of the system primary are barely visible.

The transmission line glows with RF corona as it takes current

from the top of the secondary coil (far right side of this scan,

about halfway up), to feed the base of the secondary coil. The

transmission line used here is simply a length of insulated #8

DC power cable. The jet-like sprays coming out of the trans-

mission line are locations where the discharge from the top of

the extra coil have struck the line and punctured the insulation

allowing corona leakage.

This system is operating at 1 kVA supplied by three 12 kV neon

sign transformers. The gap system consists is a single air-blast

gap. The tank circuit capacitance consists of four .02 micro-

farad homemade rolled capacitors in series/parallel for .02

microfarads system capacitance rated at about 20,000 volts

pulse. Despite the fact that this Tesla Magnifier was lashed

together with components specifically designed for 1/4 wave

Tesla work, this system is capable of 3-1/2 foot violent

discharges.

System was designed, built, and operated by Richard T. Quick II

10028 Manchester Rd., Suite 253, Glendale, MO  63122  USA