PICF1024.GIF and PICF800.GIF

These are two scans (1024x768 & 800x600) of a Tesla Magnifier

designed, built, and operated by Richard Hull. The extra coil

of the three coil system is set on top of a large HV insulator.

The extra coil is fitted with a sophisticated discharger

arraignment to protect the very low aspect ratio coil and to

reduce the incidents of downward strikes back to the trans-

mission line, which is simply an eight foot length of hard

copper water pipe. The closely coupled primary/secondary coils

(lower left) are sitting on top of a short plastic insulator to

reduce the inductive effects of the concrete floor.

This system is operating around 2 kVA, producing spark dis-

charges in excess of six feet in length. The tank circuit

capacitance consists of four homemade .018 microfarad caps

rated around 10 kV pulse, with these capacitors wired in

series/parallel so that the measured tank circuit capacitance

is .018 microfarads rated @ 20 KVAC pulse. The power supply is

a three kVA potential transformer ballasted with both resistive

and inductive elements. The spark gap system consists of a large

rotary gap specially designed for the extremely fast quenching

required on Tesla Magnifier systems.