PICK1024.GIF and PICK800.GIF

These two scans (1024x768 and 800x600) show a "bargin basement"

coil producing about twelve inches of spark. The secondary coil

is wound with enamel covered magnet wire on a four inch diameter

thin wall PVC plastic drain pipe. The plastic pipe was sealed with

several coats of polyurethane before the coil was wound, and the

coil was coated again with several coats of sealer over the

windings to reduce corona loss. The discharge terminal is an

inverted stainless steel measuring cup that lacks a handle. The

discharger is taped to the top of the secondary with several turns

of black electrical friction tape.

The primary coil is eight turns of three-wire electrical extension

cord that has all three conductors fused at the ends. The eight

turn primary is a "flat pancake" (Archimedes spiral) wound on a

coil form constructed of plexiglas strips mounted in a simple

hardwood frame. Pegs cut from wooden dowels are plugged into holes

drilled in the plexiglas strips to act as spacers between primary

turns.

The spark gap is a cylinder static gap constructed from 14, two

inch long sections of 1-inch diameter hard copper water pipe

mounted inside of a short length of standard six-inch PVC drain

pipe. Air flow through the pipe is provided by a small muffin fan.

Only seven gaps are used, with the spacing between electrodes set

at about .030 inches.

The capacitors used in this coil project are homemade salt-water

capacitors constructed using stacks of five gallon polyethylene

plastic pickle buckets filled with brine. The designer/builder

stated to me that he was very pleased with the capacitor

performance vs. time and money, and indicated he has operated

these capacitors at power levels up to 8 kVA using a reversed

power distribution transformer running 15 kv into the tank circuit

for extended periods without failure. He noted the salt water has

a very high heat capacity which prevents hot spots from forming.

He also stated the salt water appeared to boil at higher power

levels due to the electrical pulsing (not heat) and that a heavy

layer of cheap motor oil over the exposed brine reduced the corona

losses.

The power supply for the scanned photo here was a pair of 12 kv

neon sign transformers in parallel. No voltage control was used.

This coil was designed, built, and operated by: Dave Hartwick

191 Highland Rd., Chambersburg, PA  17201  USA