PICL1024.GIF and PICL800.GIF

These are two scans of my friend Rex and I setting up my ten

inch diameter secondary coil for a test fire with the new

roughed in primary coil. Rex wears the white lab coat, I wear

the jeans and tan shirt.

Above my head the old primary (8.8 turns of 1/2-inch soft copper

refrigerator tubing wound in an inverse conical section) is

hanging on the wall, the Tesla coil with discharger at the right

is set up with the new primary. The new primary is a flat pan-

cake coil (Archimedes spiral) set up on plexiglas "combs" to

insulate and space the coil turns. A large primary such as this

(15 full turns wound out of a single 100 foot length of tubing)

takes time to "train" into the new shape. The primary coil is

held in place with about a dozen nylon wire ties which can be

removed and replaced as the coil adjusts to the coil form. After

winding a primary such as this, it can take weeks of adjustments

before the coil is tamed and ready to glue into final position.

Another point to make are the loops of high pressure hose on the

wall. I use LOTS of high pressure air from a 3-1/2 horse-power

compressor w/350 gallon tank to run compressed air quenched

spark gaps.

Rex and I are working on connecting all fourteen of my homemade

rolled polyethylene/sheet aluminum/oil capacitors. We are

setting up to fire Tesla's balanced tank circuit. This means

connecting and measuring the value of two separate banks of

seven capacitors in parallel. Capacitor tanks (white and green

cylinders) are swapped back and forth until both banks measure

within .0001 microfarads of each other; both parallel banks are

then wired in series, one bank on either side of the primary

coil, for a total capacitance of .0665 measured in the tank

circuit.

The secondary coil sports a 20x5 inch commercial spun aluminum

toriod discharger, the last one I will ever buy. The toriod is

set up on two sections of PVC pipe for a stand-off insulator.

The secondary coil form has no holes drilled, the wire is simply

air wound from the top turn to the center of the toriod dis-

charger.

This system was designed and built by Richard T. Quick II,

10028 Manchester Rd., Suite 253, Glendale, MO  63122  USA