================================================================

EBONE 92

========

Contents

1     INTRODUCTION

1.1  Background

1.2  Strategy for a Shared Infrastructure

1.3  Acceptable Use

1.4  The Two Step Approach

2     EBONE 92 - THE SHORT TERM STEP

2.1  A model for a network of networks

2.2  The Ebone 92 Boundary

2.3  The Ebone 92 Services

2.4  Topology

2.5  Ebone 92 Initial Configuration

2.6  The Ebone 92 Organisation

3     THE EUROPEAN BACKBONE - IN 1993 AND BEYOND

3.1  Preparation for 1993

3.2  The Call for Tender

1    INTRODUCTION

1.1  Background

One of the most urgent actions for the European Research

Networking community is to progress the establishment of a well

managed pan-European multi-protocol backbone service. The

recognition of this need was expressed by the RARE CoA in its

May 1990 meeting in Killarney and documented in the EEPG report

tabled to the CoA in May 1991 in Blois. The EEPG report

confirmed the urgent need for 2Mbps bandwidth and tabled two

options for the technical preparation of the multi-protocol

backbone itself:

- X.25 with embedding of other layer 3 protocols;

or   - TDM with native support of layer 3 services.

An ad hoc task force was initiated by SURFnet to work out a plan

of action to implement the second EEPG strategy, concentrating

on the technical and operational aspects of the backbone. The

start-up meeting was held in Amsterdam on 2nd September 1991.

The task force reviewed the EEPG strategy, drew up an outline

"acceptable use" statement, and recommended a two step approach:

o    creation and growth during 1992 of a kernel

backbone by combining and enhancing existing

facilities;

o    merging the resultant backbone into the RARE

Operational Unit plans for 1993.

1.2  Strategy for a Shared Infrastructure

This strategy as laid out in the EEPG report treats the

provision of networking services based on a shared

infrastructure as the key issue and recognises that the

underlying technology to provide the transmission facilities

is a rapidly evolving area and must be managed as a process

of continual change. The European telecomms regulatory

environment may also exhibit some developments that a well

orchestrated research community could play to its advantage.

The approach is to evolve the currently available facilities

into a shared and managed infrastructure for the European

research network community. It is recognised that a range of

carrier level protocols are in heavy use and developments in

technology such as ATM and Frame Relay will provide

increasingly flexible methods to manage bandwidth and share

the expensive transmission facilities between services.

Thus management attention can be divided between:

o    Provision of Services      (OSI Layer 3)

o    Bandwidth Management       (OSI Layers 1-2)

The Ebone 92 strategy is to start the provision of parallel

Layer 3 services on a backbone NOW in such a way that future

technology can be "slid underneath". The benefits are

savings in line costs through better use of high speeds

(cheaper per bit) and better service through a redundant

topology linking both European and USA networks.

1.3  Acceptable Use

The target group for Ebone 92 is all the national and

international networks and international research

institutions (hereafter referred to as Regionals) providing

network services for users at higher education and research

sites, including those involved in industrial research.

In principal, the Ebone 92 will have no restrictions on

traffic. It will be up to participating networks to restrict

traffic according to their own norms.

1.4  The Two Step Approach

The two step approach generates two streams of activity: short

term implementation in 1992 and a parallel tender/procurement

process aimed at 1993.

o  Ebone 92 - The Short Term

This document defines and details proposals for the

immediate establishment of a common managed interim

pan-European Internet IP backbone based on the current ad hoc

Internet IP backbone infrastructure(s). Recommendations also

include the provision of pilot CLNS services in 1992. The

exclusion of X.25 at this step is not because X.25 is

considered less important than IP: the pan-European

provision of X.25 is well covered already by the current

COSINE IXI service and its planned enhancements.

Chapter 2 details Ebone 92.

o  1993 Multi-protocol Backbone

An immediate action is recommended for RARE to prepare a draft

Call for Tender document for a pan-European Multi-protocol

Backbone Service to be in operation by the beginning of

1993. The execution of this step is to be the responsibility

of the Operational Unit currently under establishment.

Chapter 3 outlines the planning for 1993.

2    EBONE 92 - THE SHORT TERM STEP

2.1  A model for a network of networks

Ebone 92 is to be a multi-protocol high-bandwidth pan-European

backbone for a network of networks. This model describes

Ebone 92 in terms of its boundaries:

- the functional components of the boundary;

- the services available at the boundary;

in terms of its topology:

- the connectivity;

- the redundancy;

and presents the organisational elements:

- the management and operational functions and

interfaces;

- the funding mechanisms.

2.2  The Ebone 92 Boundary

The boundary of the Ebone 92 is described in terms of the

functionality of the systems at either side and the

specifications of their common interface.

The defined functions are handled by the Ebone 92 Boundary

System (EBS) and the Regional Boundary System (RBS). See

Fig. 1.

EBONE

----------------------------------

!    backbone traffic routing    !

!                                !- Ebone Boundary System

!  access to/from the backbone   !

----------------------------------

========================     <- Ebone Boundary Interface

----------------------------------

!  access to/from the Regional   !

!                                !

!        access control          !- Regional Boundary System

!                                !

!    regional traffic routing    !

----------------------------------

REGIONAL

Figure 1:  The Ebone 92 Boundary Interface and Connected Systems

The Ebone 92 Boundary Interface is defined for Layers 1 & 2,

for routing as appropriate for the Layer 3 services, and for

network management procedures.

The Interim Specifications for the Ebone 92 Interface for IP

and CLNS working is:

Layers 1 and 2

- Ethernet (AUI Male Port provided by the EBS)

- FDDI (Dual Attachment)

- V.35 or HSSI with PPP (RFC 1171) or Frame Relay

(CCITT I.122) encapsulation

Routing

- IP with BGP (RFC 1163) or static routes

- CLNS with IDRP (ISO CD 10747) or static routes

Management

- SNMP (RFC 1157) with MIB-II (RFC 1213)

The Boundary between Ebone 92 and the connected Regionals can be

conceptually viewed as in Fig. 2. The physical connection

between the EBS and RBS can be a local link, a national wide

area link, or an international wide area link. These last two

could be called "tail" links.

------

.........     |       \

.         .    |        \    ...........

.          R#E /          \ E#R          .

. Regional  B#Bo    Ebone   oB#B  Regional .

.          S#S \          / S#S          .

..........     \        /    ..........

|      /

\ --/   ...........

o    ..            .

EBS###RBS  Regional  .

|    .             .

/|   .............

|

###

|      "Tail" link

.RBS....

.        .

. Regional .

.........

Ebone      European Backbone Network

Regional   Regional Wide Area Network

EBS        Ebone Boundary System

RBS        Regional Boundary System

###        the Ebone/Regional boundary

Figure 2:   Ebone 92 - joining the Regional Networks

2.3  The Ebone 92 Services

The EBONE 92 access modes are:

- Internet IP

- ISO-CLNS (Pilot)

These services can be considered as pins on a

"multi-protocol socket".

The interim "Ebone 92 Socket" is shown in Fig. 3.

!========================!

!                        !

!    O             O     !

!                        !

! Internet IP   ISO-CLNS !

!                        !

!========================!

Figure 3:  The "Ebone 92 Socket"

In general, service specifications will be those in common usage

in the research community, e.g.:

- Internet IP as per Internet standard for IP in RFC 791

et seq;

- ISO-CLNS as per the specifications used by the

RARE/COSINE CLNS Pilot Project.

2.4  Topology

The Ebone 92 topology will be developed in two phases.

The goal of Phase 1 is to establish an Ebone kernel by

combining existing facilities at existing network sites into

a coherent design that will exhibit the benefits of a

backbone by providing practical connectivity and high

availability both pan-Europe and to the USA. It is to be

achieved with minimal upgrading of existing "available"

lines and node equipment.

Phase 2 grows the Ebone by incorporating other sites into

the backbone. Each new backbone site must have at least two

links to existing Ebone sites (EBS to EBS links) so that a

robust mesh topology is formed. Single "tail" links can extend

Ebone access to regions (EBS to RBS links).

2.5  Ebone 92 Initial Configuration

A robust Ebone 92 kernel must provide:

-    a three node (triangle) or four node (diamond) basic

topology;

-    two links to the USA.

Simple symmetry denies the triangle and suggests a balanced ring

structure to achieve a robust kernel:

<-----------o

/ \

/   \

o     o

\   /

\ /

<-----------o

This diamond principle could lead to the following initial

configurations. Taking the map of the higher capacity European

Internet IP international links and transforming this to a

diamond could give the following initial kernel:

USA   <-------Stockholm

/        .

/           .

Amsterdam     Hamburg

\         /

\       /

USA   <--------Geneva

Adding the regional networks and some readily achievable

extensions could give:

USA   <----Stockholm (NORDUnet)

/      .

/        .

/          .

(IXI)--A'dam (SURFnet)   Hamburg (German nets)

\         /

\       /

\     /

USA   <-----Geneva (CERN) --- (SWITCH)

|

|

Bologna (GARR/INFN)

Key:  -----  existing line

.....  new line

All links are targetted to be at least 512 Kbps except the IXI

link at 64 Kbps. A new line from Stockholm to Hamburg is needed

to complete the diamond.

The Phase 1 configuration will be decided by the initial

partners in the Ebone 92 Consortium. During Phase 2 in 1992, new

partners may join at any time. Potential candidates are all

networks providing services to researchers.

2.6  The Ebone 92 Organisation

An ad hoc consortium approach is proposed to carry through the

Ebone 92 initiative. A meeting for those organisations

interested in the formation of a consortium will be held on 26th

September at CERN Geneva.

The following activities will be implemented by interim groups

drawn from the consortium of contributing organisations:

o  Getting it going

An Ebone Action Team (EAT) will be needed to prepare and

implement the initial robust Ebone kernel. This group will

consist of a "network" of today's primary junctions management

staff.

The expansion of the infrastructure will continue to be managed

by the EAT - which itself will grow as further Regionals

subscribe to Ebone 92 - until handover or incorporation into the

OPUNIT/1993 Backbone organisation.

New Ebone access points will be registered during 1992. This

information and traffic statistics will be made available to

the OPUNIT for planning transition to contracted services.

o  Operating the backbone

An Ebone Operations Team (EOT) will be needed to provide day to

day operation of the backbone services and to provide a central

point for incident handling. Initially one support person or

Full Time Equivalent (FTE) is needed for this function, to be

supplied by consortium members. Each attached Regional will be

required to nominate a contact person to liaise with the EOT.

o  Clearing house

An Ebone Administration Unit (EAU) will be needed to balance out

the costs and contributions. SURFnet BV has offered to

contribute such facilities for the 1992 interim period.

The initial consortium members need to agree on the division of

costs for the 1992 situation.

3    THE EUROPEAN BACKBONE - IN 1993 AND BEYOND

3.1  Preparation for 1993

A full multi-protocol service is required to be up and running

at the beginning of 1993 provided by a contracted service via a

Call for Tender procedure. The rational behind this step is that

the scope of the backbone service together with good procurement

practices necessitate a proper tender process. There is an

urgent need for a group to start work on this activity. It is

recommended that a RARE Task Force be charged with preparation

of the Call for Tender and evaluation criteria for the 1993

Backbone.

3.2  The Call for Tender

The Call for Tender document will specify requested services in

terms of technical and functional specifications of the services

to be supported (notably X.25, Internet IP and CLNS), the

required Quality of Service parameters and the location and

capacity of the access points.

The design and implementation details of the backbone service

will be at the discretion of the Tenderers. The depth and

breadth of contracting out services is to be discussed during

the tender process.

================================================================

END OF DOCUMENT