T h e   8 0 5 2   D e b u g g e r

(0) OVERVIEW

This is a rudimentary debugger that can be used with your 8052

microprocessor.  It can probably also be used with the other members of the

MCS-51 family, but you should probably make appropriate changes in the software

instead.  It has the following requirements:

(1) Addresses 4000 to 4680 (hex) in the 8052's code space

must be available for use.

(2) To start the interpreter, you must "call" 4000 hex.

(3) You must set up the external interrupt 0 handler so that

it pushes PSW on the stack and then jumps to address 4003 hex.

Many 8052's come packaged with an internal BASIC interpreter, already

compatible with the requirements listed above, so that you may not need to

do anything at all.  The routines needed to allow you to run this debugger from

reset are included at the top of the assembly program inside comments. If you

are going to be using this in a stand-alone environment (without a BASIC

interpreter) then uncomment these routines.

An even better idea: you may want to try your hand at integrating the

multitasking library with the debugger in order to produce something that will

run in a stand-alone environment.

The description below serves 2 purposes:

(1) To provide you with the information needed to write your own

client for this debugger

(2) To allow you to use the debugger directly, without an elaborate

client.  This will be awkward to do, though.

If you have a PC-compatible with VGA, you will be able to use the client I

have provided.  Otherwlse, you will still be able to take a large portion of

it and port it to whatever machine you are using, if you are willing to write

your own.

(1) COMMANDS and CODES

Every command sent to the debugger has the following format:

Letter  [Hex Number [Hex Number]]

The hex numbers, may be 2 or 4 digits long.  A hex digit is any of the digits

0-9, or letters a-f, or A-F.  The commands are listed below with their codes.

C o m m a n d   S u m m a r y

M, N ..... 2 digit hexadecimal numbers

MM, NN ... 4 digit hexadecimal numbers

a         Display REGS R0-R7

b         Display SFR's

c M  N    Display M bytes of DBY from N.

d MM NN   Display MM bytes of CBY from NN.

e MM NN   Display MM bytes of XBY from NN.

f MM NN   Hex Dump MM bytes of CBY from NN.  Rounds up

MM to multiple of 16.

g         Hex Load to XBY.

h NN      Call NN

i NN      Jump NN

j         Disassemble

k         Single Step

l         Exit

(a) Display REGS R0-R7

The registers R0, R1, R2, R3, R4, R5, R6, and R7 are listed with their

current values.

(b) Display SFR's

All the Special Function Registers for the 8052, which include the following:

Ports:           P0, P1, P2, P3

General Purpose: A, B, DPTR, SP, PSW

Peripheral:      TIMER0, TIMER1, TIMER2, RCAP2

Control:         TCON, TMOD, T2CON, SCON, PCON, IE, IP

(c) Display M bytes of DBY starting at N.

DBY, here, refers to the internal data address space of the 8052.  The lower

128 bytes are accessible directly by address or indirectly through the register

pointers @R0, @R1, or the stack pointer.  The upper 128 bytes are only

indirectly accessible, and tend to be used by programmers as stack space.

The debugger accepts 2 digit hexadecimal numbers for N and M.  If N + M is

100 hex, or over, the display wraps around to address 0.

(d) Display MM bytes of CBY starting at NN.

CBY refers to the code address space of the 8052.  What this address space

is actually used for depends on the circuit board holding the 8052.  It is

usually mapped by circuitry to read-only memory containing stand-alone 8052

software, or tables.  Generally, though it can be mapped to anything with the

appropriate circuitry, including to the XBY address space (described below).

The debugger accepts 4 digit hexadecimal numbers for NN and MM.  Like above

if this sum exceeds the limit (this time: 10000 hex), the display wraps around

to address 0.

(e) Display MM bytes of XBY starting at NN.

XBY refers to the external data address space of the 8052.  Unlike CBY, the

8052 language has instructions that allow you to do both input and output at

addresses in the XBY space.

This space can also be configured for anything with the appropriate

circuitry.  It is typically mapped to RAM chips where variables and other

changeable data is held for 8052 software.  Peripherals can also be mapped

into this address space with the right circuitry: for instance keypads,

keyboards, liquid crystal display registers, CRT controller registers, etc.

The format for NN and MM as the same as with the CBY display.  Addresses

will wrap around to as as above.

(f) Hex Dump MM bytes starting at NN.

Transfers MM bytes of CBY memory starting at address NN to the Client in

Intel Hex Format.  The format for the numbers NN and MM are the same as in the

previous command.  The number of MM is rounded up to the nearest multiple of

16 -- it was far easier to program it that way and it does no harm.

(g) Hex Load

Transfers data from the Client to XBY memory in Intel Hex format.  Since

the addresses are part of the format, they do not need to be explicitly

specified in the command itself.  So the command only consists of one letter.

(h) Call NN

Executes a single call instruction on address NN.  The number NN has the same

format as in the previous command.

(i) Jump NN

Executes a single jump instruction to address NN.

(j) Disassemble

Prints out 1, 2, or 3 bytes representing the binary for the next instruction

about to be executed.

(k) Single Step

Executes the next pending instruction

(l) Exit

Exits the debugger and resumes control starting at the next pending

instruction.